Patenty

Najít předchozí grafiku

Diskutovat o tomto patentu

Zobrazit PDF

Stáhnout soubor PDF


 

	Číslo publikace
	US3989994 A

	Typ publikace
	Schválení

	Číslo žádosti
	US 05/495,960

	Datum publikování
	2. listopad 1976

	Datum podání
	9. srpen 1974

	Datum priority
	9. srpen 1974

	Vynálezci
	William C. Brown

	Původní nabyvatel
	Raytheon Company

	Exportovat citaci
	BiBTeX, EndNote, RefMan

	Citované patenty (11), Citováno v patentech (41), Klasifikace (16)

	

	
	

	Externí odkazy: Databáze patentů USPTO, Databáze postoupených patentů USPTO, Espacenet

	
	


Space oriented microwave power transmission system 
US 3989994 A
ABSTRAKT
A combined active phased antenna array for satellite solar power station systems includes a plurality of directly coupled microwave energy generators for conversion of the dc power generated by a system of solar energy conversion means to microwave energy. The microwave power is transmitted through space in a narrow beam for capture and rectification at the receiving point. The microwave energy generator devices embodied in the invention are of the crossed field amplifier type having a very high efficiency and extremely long life through the use of a cold cathode secondary emitting electron source. The microwave generated power is transmitted through slotted waveguide radiator arrays with the phase of the energy monitored and corrections are made by phase shifting devices where necessary.

OBRÁZKY(2)



 




 

NÁROKY(4)
I claim:

1. In a space oriented system and apparatus for converting solar energy into electrical power including means for converting dc energy into microwave energy, an active phased antenna array for wireless transmission of said microwave energy as a microwave beam directed to earth-based collection and electrical power conversion means comprising:

a plurality of wave transmission line means having energy radiating elements;

an array of a plurality of cascaded dc-to-microwave energy converters adapted to become operative upon injection of a microwave input signal to produce an amplified output signal at a predetermined microwave frequency; and

means for directly coupling each of said energy converters to said transmission line means to radiate a microwave beam having a predetermined phase front pattern into free space.

2. A space oriented transmitting antenna array according to claim 1 wherein said wave transmission line means comprise slotted waveguide resonators with said slots being disposed in the earth-facing waveguide walls.

3. A space oriented transmitting antenna array according to claim 2 wherein said means for coupling said energy comprises a probe element extending through the unslotted waveguide walls into said transmission line means.

4. A space oriented transmitting antenna array according to claim 1 and means disposed in combination with subarrays of said energy converters and transmission line for monitoring and controlling the phase front pattern of said transmitted microwave beam comprising means for receiving a microwave energy signal having a predetermined phase to drive said array of cascaded energy converters, means for electronically comparing said received microwave signals with a reference phase signal representative of the phase of the energy converter output signals and phase-shifting means for corrective action.

POPIS
BACKGROUND OF THE INVENTION

1. Field of the Invention

The invention relates to a system and apparatus for the conversion of solar energy into microwave energy and transmission of such microwave energy through space to a receiving station.

2. Description of the Prior Art

Electrical power has become such an accepted form of energy that the demands have increased at such a rapid rate requiring the exploration for new sources. Unfortunately, present methods of generating electrical power pollute the environment and consume natural resources at a prodigious rate. New technologies have arisen with respect to the transformation of the solar energy directly into electrical power without the necessity of a thermal cycle or any moving parts and with potentially high reliability and long life. Another new technology involves the space program which permits the orbiting of large solar arrays to be illuminated by the sun. The dc power derived from the solar cells can be converted into microwave power and large amounts of such microwave power may be beamed efficiently over long distances of free space by means of a microwave beam. The carefully controlled microwave beam is captured and the microwave energy is rectified at the receiving station. The microwave beam transfer efficiencies run as high as 90 to 95%. To transmit the energy through the earth's atmosphere with relatively small attenuation due to moisture the use of a wavelength of approximately ten centimeters is desirable. The size of the microwave beam apertures necessary for efficient transmission is sufficiently large so that very high power handling capabilities is possible. Passive radiation of waste heat in the microwave energy transmitters is made possible by the large antenna array area and the use of highly efficient microwave generators. For the purposes of the specification the term "micorwaves" refers to energy in that portion of the electromagnetic energy spectrum having wavelengths in the order of 1 meter to 1 millimeter and frequencies in excess of 300 MHz.

The conversion of microwave energy to electrical power has been disclosed in prior art embodiments utilizing rectifier antenna (rectenna) arrays comprising a considerably large number of solid state semiconductor devices such as Schottky-barrier diodes together with half-wave dipoles each terminated in a semi-conductor diode means. A complete rectenna array is disclosed in U.S. Pat. No. 3,434,678 issued Mar. 25, 1969 to W. C. Brown et al and assigned to the assignee of the present invention. The wireless transmission of the microwave energy via beams is preferred due to the relatively high power density which can be focused upon a relatively small area at the receiving location. In accordance with the laws of optics the sharpness of the microwave beam varies as the ratio of antenna dimensions to the wavelength of the transmitted energy. Accordingly, for a given predetermined power density and beam sharpness very short wavelengths of microwave energy facilitates a corresponding decrease in the dimensions of the antenna means for transmitting and receiving. In addition, in long wave or radio-type transmission, difficulties are encountered as a result of natural or man-made interference and noise and this problem is considerably reduced with the transmission of microwave energy. The recent United States space program has demonstrated that the employment of very short microwaves for beamed energy is preferable for penetration of certain atmospheric reflection layers together with the advantages of accurate focusing and high power density of the transmitted energy.

Further examples of the utilization of microwave energy in space applications may be found in U.S. Pat. No. 3,174,705, issued Mar. 23, 1965 to D. Schiff et al, as well as 3,083,528, issued Apr. 2, 1963 and 3,114,517, issued Dec. 17, 1963, both to W. C. Brown. The foregoing patents are also assigned to the assignee of the invention and disclose various conversion means utilizing heat exchangers for conversion of mircowave energy into electrical energy or heat which can be utilized for directly or indirectly propelling a space station.

An example of the satellite solar power station concept is found in U.S. Pat. No. 3,781,647, issued Dec. 25, 1973 to P. E. Glaser. The satellite system includes several geostationary satellites positioned in an energy-receiving relationship to the sun. The solar energy conversion means include guidance and control means to correctly orient the energy conversion means to the source. The large solar cell array is located in an equatorial synchronous orbit around the earth where it is illuminated by the sun with a duty cycle in excess of 99%. The dc power derived from the solar cells is converted into microwave power and beamed to earth.

A most efficient superpower high frequency microwave energy generator operative in the desired band is the Amplitron which has a broad bandwidth and very high efficiency with extremely long life. The electron tube comprises a nonreentrant, non-resonant slow wave propagating structure having two terminations and a continuous cathode capable of emitting electrons from a major portion thereof. Cold cathodes having a high secondary electron emission ratio are particularly desirable in view of the absence of any requirements for power supplies for directly heating oxide-type cathode emitters. The illustrative superpower devices are capable of producing 15 to 20 kilowatts of average power at the desired frequency with capabilities in the region of 500 kilowatts or more of average power and 50 megawatts peak power. A complete description of the Amplitron microwave amplifier devices may be found in U.S. Pat. No. 2,673,306, issued Mar. 23, 1954, 2,859,411, issued Nov. 4, 1958, 2,933,723, issued Apr. 19, 1960 and 2,977,502, issued Mar. 28, 1961 all in the name of W. C. Brown and assigned to the assignee of the present invention. An example of a secondary emissive cold cathode structure is found in U.S. Pat. No. 3,096,457 issued July 2, 1963 to W. A. Smith, Jr. et al.

The current energy crisis throughout the world due to the growing shortage of the world supplies of fossil fuels, such as coal, oil and natural gas due to the increases in the level of consumption has made the search for new sources of energy imperative. The use of solar energy has now been proven in the space program and satellite solar power stations can be developed, hopefully, before the turn of the century. Solar energy offers an almost unlimited source for man's energy needs and the devices for conversion of the dc energy to microwave energy have expected efficiencies of approximately 85 to 90%. The direct coupling of the microwave energy sources to the transmitting phased antenna array should also result in expected transmission efficiencies in the 85 to 95% region. In acordance with the teachings of the referenced satellite solar power station U.S. Pat. No. 3,781,647 the actual size of the solar cell area for collection and conversion of the solar energy would be approximately 25 square miles. The transmitting phased antenna array would be approximately 0.6 of a mile in diameter, assuming a transmitting wavelength of approximately 10 centimeters. A transmitting antenna, illustratively, is designed to provide approximately 10,000 megawatts of beam power which could be supplied by 10,000 microwave energy generators, in 100 rows and 100 panels in each row with each tube having a 1 megawatt output. The present invention achieves the unique arrangement for an active phased antenna array with mircowave generators to provide for the maximum efficiency of the solar power station. Eventually solar power stations having a 200 square mile area would require one to two million microwave tubes for each station.

SUMMARY OF THE INVENTION

In accordance with the present invention a system is provided incorporating a large number of microwave frequency energy converters in an active phased transmitting antenna array with transmission line means coupled to each of the energy converters. An exemplary device the Amplitron utilized has a cold cathode of a pure metal secondary electron emissive material and the combined energy radiated from the transmitting antenna array is transmitted through space in a relatively narrow beam for capture at an earth station. The phased array antenna incorporates waveguide transmission lines with radiating slots and phase comparator means, as well as, phase shifter means in each subarray of Amplitron amplifiers to provide phase coherency in the transmitted signals. The elimination of the heater supplies for the cathodes through the utilization of the cold cathode structures results in substantial savings in weight as well as cost of the satellite solar power station. The cold cathode devices are activated by a coupled RF injected signal which can be generated from another generator source on the station or be fed through space from an earth location.

The passive radiation of generated heat in space is made possible by the large phased antenna array with the direct coupling to the wave transmission line means which results in reduction of waveguide skin losses by reducing the lengths of the waveguide runs in each array. In the space-oriented environment further economies in space, as well as, cost are realized since in the space applications no vacuum envelope is required enclosing the components of the Amplitron converter. The microwave windows and vacuum-tight joints in the envelopes are a major consideration in the cost and weight of such energy converters utilized in the earth environment.

Microwave power generation efficiencies of 85-95% may be realized with the disclosed converters through the utilization of the phased transmitting antenna array and with waveguides directly coupled to each of the energy converters transmission efficiencies of 94% are expected.

BRIEF DESCRIPTION OF THE DRAWINGS

Details of an illustrative embodiment of the invention will be readily understood after consideration of the following description, with reference directed to the accompanying drawings, wherein:

FIG. 1 is an isometric view of a portion of an overall active phased transmitting antenna array embodiment of the invention;

FIG. 2 is a diagrammatic representation of an exemplary microwave energy converter, namely an Amplitron, illustrative of the principles of operation;

FIG. 3 is a schemmatic view of a portion of a subarray of the overall active phased array antenna embodying the invention; and

FIG. 4 is a block diagram illustrative of the cascade arrangement of amplifying and radiating elements of the invention.

DESCRIPTION OF THE PREFERRED EMBODIMENT

Referring to FIG. 1 the active phases transmitting antenna array 10 is shown supported in space as a part of the satellite solar power station with the earth 12 partially visible through the cut away portion of the antenna. A large number of microwave energy converters, such as the Amplitron are illustrated in a subarray comprising a plurality of waveguide resonator transmission line means 14 having radiating slots 16 in one of the broad walls. In the satellite solar power station concept the radiating slots are oriented in the earth-facing surface to direct the microwave energy beam to the receiving location located on earth 12. The Amplitron microwave energy converter 18 is shown coupled to the slotted waveguide resonators 14 by means of probe member 20.

Before proceeding with a detailed description of the phased array antenna, the microwave energy converter 18 will be described with reference being directed to FIG. 2. The operating principle of crossed-field devices, such as the Amplitron, is based on synchronous interaction between a reentrant electron beam originating from a continuous cold cathode 22 of a pure metal and RF energy propagating along a slow wave circuit provided by anode 24. Such devices are operated by supplying a static magnetic field parallel to the axis of the cathode and an electric field provided between the cathode and the RF circuit by voltage source 26. The direction of the magnetic field is illustrated by the X 28 and is provided by any well known means including the permanent magnets of such new materials as samarium-cobalt. The RF circuit 24 is nonreentrant unlike the electron beam, and the characteristic impedance of the RF circuit is matched and terminated at both ends by external RF connections, an input 30 and an output 32, over the frequency region desired.

A microwave signal 34 is injected at the input end 30 of the RF circuit. The interaction between the microwave energy on the slow wave propagating circuit 24 and the adjacent electron beam results in a rotating spoke-like space charge 36 which rotates counterclockwise as indicated by arrows 38 while the RF energy propagates clockwise along the circuit as indicated by arrows 40. The rotating spokes of space charge induce currents in the RF circuit and provide for amplification of the input microwave signals 34 as an amplified signal 42 at output terminal 32 which is coupled to probe member 20 illustrated in FIG. 1 of the transmitting antenna array. Efficiencies of between 80 and 90% have been obtained with the use of the crossed-field Amplitron energy converter. Efficiency values are obtained from the equation efficiency = microwave output-microwave input/dc power input. The long life expected with the use of pure metal cold cathodes, such as platinum for the secondary electron emission along with the absence of the requirement for an evacuated envelope make such devices an attractive microwave energy converter for use in satellite solar power stations. Further details relative to the amplifier may be found in "Crossed-Field Devices," Volume 2, E. Okress, editor-in-chief, Academic Press, 1961, in the chapter by W. C. Brown, pages 165-209.

Referring next to FIGS. 3 and 4, a typical subarray in the phased transmitting antenna array is schematically illustrated. The gridded square 44 represents diagrammatically the overall transmitting antenna array 10. The portion or subarray illustrated is an exploded view of the components disposed within the circle 46 which will now be described. As previously stated, an exemplary antenna array could cover an overall area of 25 square miles for up to 10,000 Amplitron microwave energy converters. Each subarray is provided with means for comparing the arrival phase of a pilot beam from the earth with a reference phase and establishing a low level resultant signal with the appropriate phase. The power level of this signal is then amplified by suitable means to a level appropriate for driving the first Amplitron microwave energy converter. This in turn drives several other Amplitrons which in turn drive the string of cascaded Amplitrons. The phase of the microwave power in the slotted waveguide resonator transmission means is monitored throughout the subarray and corrections in phase are made by the appropriate phase shifting devices.

Referring to FIG. 4 the principle of operation of the overall transmitting antenna including the array of cascaded microwave converters commences with the RF drive signal 48 which is fed to the RF circuit and may comprise a pilot beam from an earth station to establish a reference phase for the overall transmitting antenna. The first Amplitron device 50 becomes operative and drives successive cascaded Amplitron devices 52 and 54 and this arrangement is repeated many times in the overall antenna array. Radiating elements 56, for example, the slotted waveguide resonator transmission line means 14, 16 form the overall beam transmitted from space to the receiving location.

Referring next to FIG. 3 the antenna components are illustrated comprising amplifiers 58, designated by the circles, with coupled radiating elements 60 and intermittently disposed phased comparator and corrective phase shifter means 62 designated by the square box. Centrally disposed and linearly oriented with respect to all of the cascaded amplifiers, antenna radiating elements and phase comparison and shifting means is a central signal source and waveguide transmission means 64 including an RF frequency reference which is either space-fed or a microwave frequency generator mounted in the array. The RF frequency is fed to an electronic phase shifter having a master phase reference. A high gain preamplifier feeds this signal to the first Amplitron device and then to the complete subarray cascaded amplifiers. In this manner a highly efficient microwave beam transfer efficiency may be realized which can run as high as 94%. Large amounts of converted power at microwave frequencies may be efficiently transferred over extremely long distances in free space by means of the transmitted microwave beam formed by the large antenna array incorporating a plurality of subarrays similar to that illustrated in FIG. 3 and designated by the numeral 46. The heat generated by the energy conversion is efficiently radiated in space by fin radiators 66 affixed to each of the Amplitron amplifier devices as shown in FIG. 1. The heat dissipation factor is improved since the radiators radiate from both sides and the average temperature of the fin type cooling radiators approaches 300° C. With both sides of the radiators radiating effectively very low weights structures are required for cooling.

There is thus disclosed an active phased transmitting antenna array comprising a large number of cascaded microwave amplifiers, each directly coupled to waveguide resonator transmission line means to collectively form a phase-controlled microwave beam for transmission of the converted solar energy for collection and conversion to electrical energy at a receiving location. Numerous modifications, alterations and variations in the disclosed embodiment will be evident to those skilled in the art. The foregoing description of an illustrative embodiment, therefore, is to be considered in its broadest aspects and not in a limiting sense.

CITOVANÉ PATENTY
	Citovaný patent
	Datum podání
	Datum publikování
	Přihlašovatel
	Název

	US2933723 *
	2. leden 1958
	19. duben 1960
	Raytheon Co
	Low level duplexer system

	US2977502 *
	26. únor 1957
	28. březen 1961
	Raytheon Co
	Electronic discharge devices of the magnetron type

	US3096457 *
	31. březen 1959
	2. červenec 1963
	Raytheon Co
	Traveling wave tube utilizing a secondary emissive cathode

	US3432690 *
	31. srpen 1966
	11. březen 1969
	Us Army
	Thermionic conversion of microwave energy to direct current

	US3434678 *
	5. květen 1965
	25. březen 1969
	Raytheon Co
	Microwave to dc converter

	US3445748 *
	16. březen 1967
	20. květen 1969
	Zenith Radio Corp
	Microwave power rectifiers

	US3462636 *
	6. duben 1966
	19. srpen 1969
	Siemens Ag
	System for the conversion of microwave energy into electric direct current energy utilizing an electron beam tube

	US3479577 *
	19. květen 1967
	18. listopad 1969
	Raytheon Co
	Crossed field microwave rectifier

	US3518688 *
	15. září 1969
	30. červen 1970
	Itt
	Microwave strip transmission line adapted for integral slot antenna

	US3526898 *
	3. duben 1967
	1. září 1970
	Raytheon Co
	Antenna with translational and rotational compensation

	US3781647 *
	26. červenec 1971
	25. prosinec 1973
	Little Inc A
	Method and apparatus for converting solar radiation to electrical power


* Citováno examinátorem

CITOVÁNO V PATENTECH
	Citující patent
	Datum podání
	Datum publikování
	Přihlašovatel
	Název

	US4187506 *
	16. říjen 1978
	5. únor 1980
	Nasa
	Microwave power transmission beam safety system

	US4354153 *
	19. listopad 1979
	12. říjen 1982
	Litton Systems, Inc.
	Microwave oven leakage detector and method of using same to test door seal leakage

	US4490668 *
	12. červenec 1979
	25. prosinec 1984
	Rca Corporation
	Microwave radiator utilizing solar energy

	US4689631 *
	28. květen 1985
	25. srpen 1987
	American Telephone And Telegraph Company, At&T Bell Laboratories
	Space amplifier

	US5049753 *
	28. červen 1990
	17. září 1991
	The United States Of America As Represented By The Secretary Of The Navy
	Optically powered charged particle accelerator

	US5068669 *
	1. září 1988
	26. listopad 1991
	Apti, Inc.
	Power beaming system

	US5218374 *
	10. říjen 1989
	8. červen 1993
	Apti, Inc.
	Power beaming system with printer circuit radiating elements having resonating cavities

	US5260639 *
	6. leden 1992
	9. listopad 1993
	The United States Of America As Represented By The Administrator Of The National Aeronautics And Space Administration
	Method for remotely powering a device such as a lunar rover

	US5982139 *
	19. září 1997
	9. listopad 1999
	Parise; Ronald J.
	Remote charging system for a vehicle

	US6114834 *
	8. listopad 1999
	5. září 2000
	Parise; Ronald J.
	Remote charging system for a vehicle

	US6127799 *
	14. květen 1999
	3. říjen 2000
	Gte Internetworking Incorporated
	Method and apparatus for wireless powering and recharging

	US6936760
	26. listopad 2003
	30. srpen 2005
	Solaren Corporation
	Space-based power system

	US6967462
	5. červen 2003
	22. listopad 2005
	Nasa Glenn Research Center
	Charging of devices by microwave power beaming

	US7091918
	25. říjen 2004
	15. srpen 2006
	University Of South Florida
	Rectifying antenna and method of manufacture

	US7612284
	25. srpen 2005
	3. listopad 2009
	Solaren Corporation
	Space-based power system

	US7900875
	18. červenec 2005
	8. březen 2011
	Criswell David R
	Power generating and distribution system and method

	US7936095 *
	27. březen 2007
	3. květen 2011
	PANASONIC, Corporation
	Communication system using directional control of electomagnetic wave power transmission

	US8159364
	
	17. duben 2012
	Omnilectric, Inc.
	Wireless power transmission system

	US8410953
	
	2. duben 2013
	Omnilectric, Inc.
	Wireless power transmission system

	US8446248
	
	21. květen 2013
	Omnilectric, Inc.
	Wireless power transmission system

	US8558661
	27. březen 2013
	15. říjen 2013
	Omnilectric, Inc.
	Wireless power transmission system

	US8596581
	12. prosinec 2011
	3. prosinec 2013
	David R. Criswell
	Power generating and distribution system and method

	US8653784 *
	11. prosinec 2010
	18. únor 2014
	Todd Allen Bland
	System and method for relaying energy from a space transmitter to an electronic device via an earth station

	US8854176
	14. říjen 2013
	7. říjen 2014
	Ossia, Inc.
	Wireless power transmission system

	US9142973
	6. říjen 2014
	22. září 2015
	Ossia, Inc.
	Wireless power transmission system

	US9312701
	28. červenec 2015
	12. duben 2016
	Wi-Charge Ltd
	System for optical wireless power supply

	US20040173257 *
	26. listopad 2003
	9. září 2004
	Rogers James E.
	Space-based power system

	US20060038083 *
	18. červenec 2005
	23. únor 2006
	Criswell David R
	Power generating and distribution system and method

	US20060185726 *
	25. srpen 2005
	24. srpen 2006
	Solaren Corporation
	Space-based power system

	US20060201547 *
	22. únor 2006
	14. září 2006
	Solaren Corporation
	Weather management using space-based power system

	US20080000232 *
	21. únor 2007
	3. leden 2008
	Rogers James E
	System for adjusting energy generated by a space-based power system

	US20080309452 *
	14. červen 2007
	18. prosinec 2008
	Hatem Zeine
	Wireless power transmission system

	US20090146503 *
	27. březen 2007
	11. červen 2009
	Matsushita Electric Industrial Co., Ltd.
	Communication system

	US20090195084 *
	25. září 2008
	6. srpen 2009
	Todd Allen Bland
	Cellular phone and portable electronic device sun powered process

	US20100289342 *
	11. květen 2010
	18. listopad 2010
	William Eugene Maness
	Space-Based Power Systems And Methods

	US20100315045 *
	
	16. prosinec 2010
	Omnilectric, Inc.
	Wireless power transmission system

	US20110080135 *
	
	7. duben 2011
	Todd Allen Bland
	Solar Powered Charging Of An Electronic Device

	US20110204159 *
	8. prosinec 2010
	25. srpen 2011
	Solaren Corporation
	Weather management using space-based power system

	CN102437774A *
	30. prosinec 2011
	2. květen 2012
	四川大学
	High-power centralized microwave rectifier circuit

	WO1998050996A2 *
	4. květen 1998
	12. listopad 1998
	Parise Ronald J
	Remote vehicle charging system

	WO1998050996A3 *
	4. květen 1998
	4. únor 1999
	Ronald J Parise
	Remote vehicle charging system


* Citováno examinátorem

KLASIFIKACE
	
	

	Americká klasifikace
	322/2.00R, 315/39.3, 244/172.7, 343/771

	Mezinárodní klasifikace
	H01Q21/00, H02J17/00, H02M11/00, H02N11/00

	Klasifikace CPC
	H02J17/00, H02M11/00, H01Q21/005, H02N11/002

	Evropská klasifikace
	H02J17/00, H01Q21/00D5B1, H02N11/00B, H02M11/00


